      - SPRINKLERS

          - Description: Rain Bird® XLR Series Rotor designed to provide uniform coverage over entire area of spray shown on drawings at available water pressure or as follows:

              - Rain Bird® XLRADJNPT Rotor for large synthetic or natural turf areas (81-177 feet (25.6-57.3 m)) spacing: maximum 120 psi (8.3 bar).

          - Impact rotor for large synthetic or natural areas (81-177 feet (25.6-57.3 m)) spacing: maximum 120 psi (8.3 bar).  Irrigation rotor specifications include but are not limited to:

              - The rotor swing arm and nozzle shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - The rotor body and barrel shall be constructed of aluminum.

              - Reversing 20-340° arc adjustment or full 360° rotation.

              - Operating range of 81 to 177 feet (25.6 to 57.3 m).

              - Operating pressure range of 30 to 120 psi (2.1 to 8.3 bar).

              - Shall include Ø2 inch NPT inlet.

              - Shall include a water deflector that uniformly distributes water for the entire throw distance.

              - Shall accept an optional jet breaker to improve distribution uniformity.

              - Shall include an automatic brake mechanism that allows the water jet to maintain a constant rotation speed in all operating conditions independent of the flow and pressure.

              - Shall include a rotation speed selector that has three rotation speeds to choose from.

              - Shall accept nozzles with indicated flow rates from 35 to 363 gpm (7.8 to 79.2 l/m) (See Figure) and an adjustable exit trajectory of 15-45°.

              - Shall include a one-year trade warranty.

              - Shall be manufactured by Rain Bird® Corporation, Azusa, California.

Nozzle Performance Chart