      - IRRIGATION COMPONENTS AND/OR SYSTEMS

        - Rain Bird® PESB-R Series Valve

          - General Information

            - Provide Rain Bird® 150PESB-R as per the specification.

          - Rain Bird® 150PESB-R Valve

            - Rain Bird® 150PESB-R valve specifications include but are not limited to:

              - The valve body and bonnet shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - The bonnet shall be assembled to the body using multi-drive screws for use with flathead, Phillips, or hexagonal tools.

              - Shall possess a water tight seal between the body and bonnet.

              - Shall be a normally closed, forward flow design.

              - Shall have a filtered pilot flow to resist debris and clogging.

              - Shall be slow closing to prevent water hammer from causing subsequent system damage.

              - Shall have a Ø1.5 inch NPT globe inlet as well as a Ø1.5 inch NPT outlet.

              - Shall have a manual internal bleed capability to operate the valve without allowing water into the valve box.

              - Shall have an external bleed to permit flushing of debris from the system.

              - Shall have a flow control handle to adjust water flow as needed.

              - Shall include a chlorine and chemical resistant scrubber mechanism that scrapes the filter screen to break down grit and plant material to prevent clogging.

              - Shall include chlorine and chemical resistant diaphragm.

              - Shall include a 24 VAC 50/60 Hz solenoid capable of 2-wire operation.

                - Inrush current: 0.41A (9.9VA) at 60Hz

                - Holding current: 0.14A (3.43 VA) at 60Hz

              - Operating flow rate of 20-150 gpm (75.7-567.8 l/min).

              - Operating pressure range of 20-200 psi (1.38-13.8 bar).

              - Water temperature: Up to 150°F (66°C).

              - Ambient temperature: Up to 150°F (66°C).

              - Shall accommodate an optional field-installed Rain Bird® PRS-D pressure regulating dial.

              - Shall accept an optional latching solenoid for use with Rain Bird® battery operated controllers.

              - Shall include a purple flow control handle for use with non-potable water applications.

              - Shall have a heavy-duty stainless steel spring for positive diaphragm closure.

              - Shall include a five-year trade warranty.

              - Shall be manufactured by Rain Bird® Corporation, Azusa, California.

Valve Pressure Loss Chart