# PCS- 010 -020 -030 -040 -060 -090 PCS-125 -175 -260 -370

1800 Series Pressure Compensating Screens

The under-the-nozzle pressure compensating screen shall be a two-piece screen combination with a flow-control module positioned in the middle.

The pressure compensating screens (PCS Series) shall be color-coded by flow as follows:

Model      Color    GPM

PCS-010    Purple   .1

PCS-020    Brown    .2

PCS-030    Silver   .3

PCS-040    Orange   .4

PCS-060    Black    .6

PCS-090    White    .9

PCS-125    Green   1.25

PCS-175    Yellow  1.75

PCS-260    Blue    2.6

PCS-370    Red     3.7

Screen openings shall be .20".

The screen shall be compatible with all 1800 Series MPR plastic nozzles.

The screens shall regulate flow at inlet pressures of 30 to 70 psi and can be used to eliminate fogging or shorten radius.

The screens shall be as distributed by Rain Bird Corporation, Azusa, California.

A-8S

Brass Nozzle Shrub Adapter

The brass nozzle shrub adapter shall have a 1/2" (FNPT) inlet and a fine threaded outlet to accept all 1800 Series brass and plastic MPR nozzles.

The adapter shall be of brass construction and shall accept protective, non-clogging 1800 Series filter screens or pressure compensating screens (PCS Series).

The adapter shall be as manufactured by Rain Bird Corporation, Azusa, California.

PA-80

Plastic Adapter

The plastic adapter shall have a 1/2" (MNPT) outlet and a fine threaded female inlet which will accept 1800 pop-up stem, thus adapting the 1800 Series sprinkler for use with various 1/2" (FNPT) sprinklers and nozzles.

The adapter shall be constructed of rugged UV resistant thermo-plastic material.

The overall height shall be 1.5" with an exposed height of .8" above the 1800 Series cap when stem is fully retracted.

The adapter shall be as manufactured by Rain Bird Corporation, Azusa, California.

# PA-8S Plastic Shrub Adapter

The plastic shrub adapter shall have a 1/2" (FNPT) inlet and its fine-threaded outlet shall accept all plastic 1800 MPR nozzles, 1800 Series filter screens and PCS Series screens.

The plastic shrub adapter shall be constructed of rugged UV resistant plastic.

The plastic shrub adapter shall be as manufactured by Rain Bird Corporation, Azusa, California.

# PA-8S-NP Plastic Shrub Adapter

The purple plastic shrub adapter shall have a 1/2" (FNPT) inlet and its fine-threaded outlet shall accept all plastic 1800 MPR nozzles, 1800 Series filter screens and PCS Series screens.

The purple plastic shrub adapter shall be constructed of rugged UV resistant plastic.

The purple plastic shrub adapter shall be as manufactured by Rain Bird Corporation, Azusa, California.

# PA-8S-PRS

Pressure Regulating Shrub Adapter

Additional Bidding Specifications

When so indicated on the design, the plastic shrub adapter shall also include a built-in pressure regulator. The device shall regulate the nozzle pressure to 30 PSI with inlet pressure from 35 to 70 PSI. Below 35 PSI, the pressure loss shall not exceed 6 PSI.

The plastic shrub adapter with pressure regulating device shall have a 1/2" (FNPT) inlet and its fine-threaded outlet shall accept all brass and plastic 1800 Series nozzles. Its overall height shall be

5 1/4".

The plastic shrub adapter shall be as manufactured by Rain Bird Corporation, Azusa, California.

1800-VPC

1800 Series Vandal Resistant Cap

The vandal resistant cap shall have a stainless steel locking set screw machined into the side of the cap for installation on 1800 Series pop-up spray heads. It shall be capable of being installed on either new or existing systems already in the field using a 1/16" hex wrench.

The vandal resistant cap shall be constructed of heavy-duty UV resistant plastic to the same size specifications as the 1800 Series standard cap.  The cap shall be marked "VPC" for vandal proof cap.

The cap shall be as manufactured by Rain Bird Corporation, Azusa, California.

1800-EXT

1800 Series Plastic Extension

The extension shall thread on all 1800 Series pop-up spray-heads and provide an additional 6" in height.  No tools shall be required for installation.  Unique breakaway feature shall allow the extension to be reinstalled without damage to the threads.

The extension shall be constructed of heavy-duty UV resistant thermoplastic to the same specifications as the 1800 Series standard extension.

The extension shall be as manufactured by Rain Bird Corporation, Azusa, California.

# SB-Series

# Spiral Barb Fittings

The spiral barb fittings shall be used in conjunction with swing pipe as a flexible swing joint assembly for 1/2" (15/21) and 3/4" (20/27) inlet sprinklers. The fittings shall have a maximum operating water pressure of 80 PSI and 8 gpm.

The spiral barb fittings shall come in seven models to fit various needs. The fittings shall be constructed of Acetal thermoplastic material and be designed so as to permit twist-in insertion, eliminating the need for glue or clamps.

The fittings shall be as manufactured by Rain Bird Corporation, Azusa, California.

# SPX-100 and SPX-FLEX 100

Swing Pipe Tubing

The swing pipe shall be flexible black tubing constructed of linear low-density polyethylene material. The tubing shall have a wall thickness of 0.098” both for the SPX and SPX-FLEX models.  It shall have an inside diameter of 0.487”for SPX and SPX-FLEX for use with SB Series spiral barb fittings without the necessity of glue or clamps. The model number, manufacturer’s name and date of manufacture shall be printed at intervals between 3’ and 6’ along the length of the tubing. Each 18" (46 cm) length of tubing shall be capable of pressure testing at the rate of 80 pounds per square inch (psi) (5,5, bar) per second to a minimum burst pressure of 325 psi (22,4 bar) for SP and 350 psi (24,1 bar) for SPX and SPX-FLEX.  The swing pipe shall be capable of 8 gallons per minute (gpm) maximum.

The tubing shall have an operating pressure rating of 80 PSI at 110◦ F (5,5 bar at 43◦C).

The swing pipe tubing shall be as manufactured by Rain Bird Corporation, Azusa, California.

1300A-F

Adjustable Flood Bubbler

The adjustable full circle bubbler shall have a discharge rate of ___ units flow at ___ units pressure. It shall operate over a pressure range of 10 to 60 PSI (0,7 to 4,0 bar).

The bubbler body shall be constructed of durable UV-resistant plastic. It shall have a plastic inlet filter screen to protect the nozzle against clogging, and a stainless steel adjusting screw, capable of shutting off the bubbler and regulating the flow.

The bubbler shall have a 1/2" (FNPT) inlet for connecting to the piping system riser.

The bubbler shall be as manufactured by Rain Bird Corporation, Azusa, California.

1401, 1402, 1404, 1408

Pressure Compensating Flood Bubbler

The bubbler shall be a pressure compensating type capable of providing a consistent discharge rate of ___ units flow at ___ units pressure.

The bubbler shall have a “trickle” pattern (1401& 1402 models) or an “umbrella” pattern (1406 &1408 models) discharge.

The bubbler assembly shall have a plastic inlet filter screen to protect the nozzle against clogging.

The pressure compensating bubbler shall be of a permanently assembled design constructed of durable, UV resistant plastic with an integral rubber flow washer for regulating the flow rate at an operating pressure range of 20 to 90 PSI (1,5 to 6,0 bar).

The pressure compensating bubbler shall have a 1/2"(15/21) (FNPT) inlet for connection to the piping system riser.

The pressure compensating bubbler shall be as manufactured by Rain Bird Corporation, Azusa, California.