      - IRRIGATION COMPONENTS AND/OR SYSTEMS

        - Controllers

          - General Information

            - Provide irrigation controllers as per the specification.

            - The irrigation controller shall be comprised of components as outlined in this specification in order to meet the electrical demands and control requirements of the irrigation zones they service.

          - Controllers

            - The irrigation controller specifications include but are not limited to:

              - The controller shall be of a hybrid type that combines electro-mechanical and microelectronic circuitry capable of fully automatic or manual operation.

              - The controller shall be housed in a wall-mountable, weather resistant plastic cabinet with locking cabinet door.

              - The controller shall require only two mounting screws for installation.

              - The controller shall have a fixed station capacity of four, six or eight stations.

              - The controller shall have a graphic based user interface.

              - The controllers shall have an LCD display that is capable of displaying each zone’s irrigation scheduled watering days, start times, and run times in the same screen.

              - Station timing shall be from 0 minutes to 199 minutes.

              - The controller shave have independent zone-based scheduling allowing each valve to be scheduled independently.

                - The zone schedules shall have the capabilities of different start times, watering day cycles, and zone run times.

              - Each zone shall have up to six start times per day that are independent of each other.

              - The controller shall incorporate a rapid programming function which automatically copies the start times and watering days from Zone 1 to all remaining zones at the initial set up of the controller.

              - The controller shall have the capabilities of copying a previous zone to another for faster initial setup and more reliable programming.

              - The controller electrical specifications shall be:

                - Input required: 230VAC ± 10% at 50Hz.

                - Output: 24 VAC 650mA

                - Power back-ups:  2 x AAA batteries maintain time and date while nonvolatile memory maintains the programming.

              - The controller shall be programmable under battery power.

              - The controller shall have a mechanism to indicate to the user when AC-power is not present when running on the batteries.

              - The controller shall have a 365-day calendar with leap year intelligence.  The leap year intelligence allows the use of “Odd” or “Even” day watering schedule without changing the date on leap years.

              - Shall have four irrigation cycle modes for maximum flexibility and compliance to all major watering restrictions.

                - Custom 7-day calendar cycle

                - 1-14 day (Cyclic)

                - Odd cycle

                - Even cycle

              - Shall include non-volatile (100 year) program memory to maintain the irrigation schedule during a power outage or if unwanted schedule changes are made.

              - The controller shall have a seasonal adjust feature which adjust the rune time for all zones from -90% to +100% in increments of 10%.

                - Seasonal adjust shall effect all programs simultaneously.

              - The controller shall have a 12-hour AM/PM or 24 hour military (for 230VAC models) clock with a midnight day change over.

              - The controller shall have a weather sensor input with software override.

                - All stations shall have the capability of independently obeying or ignoring the weather sensor.

              - Shall include a diagnostic self-setting circuit breaker that identifies a valve or wire fault and continues to water operable stations.

              - The controller shall offer manual watering of all zones sequentially or one zone at a time.

                - When manual watering is activated; the unit shall ignore the status of the weather sensor (if connected) and re-enable the sensor when manual watering is completed.

              - The controller shall have default settings that allow the contractor to set his/her own default program.

                - Shall be used for ease in restoring a schedule that has been altered by a homeowner or to replace a temporary schedule for new vegetation.

              - The controller shall have the capacity for the installer to restore the schedule to the factory fresh conditions in order to start the programming with a blank state.

              - The controller shall be compatible with a specific irrigation control remote and have a mechanism to communicate with future expansion accessories.

              - The controller shall have a guide for ½” or ¾” conduit for a more professional installation of field wires into the cabinet.

              - The controller shall include two AAA batteries that shall maintain time and date during a power outage.

              - Shall have the dimensions of:

                - Width:	6.65 in. (16.9 cm)

                - Height:	5.90 in. (15.0 cm)

                - Depth: 	1.54 in. (3.9 cm)