      - SPRINKLERS

          - Description: Manufacturer’s standard rotor designed to provide uniform coverage over entire area of spray shown on drawings at available water pressure or as follows:

              - Shrub rotor for shrub or small turf areas (15-35 feet (4.6-10.7 m)) spacing: maximum 55 psi (3.8 bar).

          - Shrub rotor for shrub or small turf areas (15-35 feet (4.6-10.7 m)) spacing: maximum 55 psi (3.8 bar).  Irrigation rotor specifications include but are not limited to:

              - The rotor body, stem and nozzle, shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - Shall have integrated seals and method for clearing debris.

                - Oversized pressure activated wiper seal prevents leaks and protects internals from debris.

                - Ported arc adjustment screw to clear debris from unit.

              - Ratchet mechanism to allow easy arc alignment.

              - Reversing 40-360° arc adjustment through the top of the rotor requires only a flat blade screwdriver.

              - 7.0 inch (17.8 cm) body height; 4.0 inch (10.2 cm) pop-up height (measured to center of nozzle).

              - Operating range of 15 to 35 feet (4.6 to 10.7 m).

              - Operating pressure range of 15 to 55 psi (1.0 to 3.8 bar).

              - Shall have a precipitation rate of 0.37 to 0.83 in/hr (9 to 21 mm/hr).

              - Shall include check valve with up to 7 feet (2.1 m) of elevation hold up.

              - Exposed surface diameter shall measure 1.3 inch (3.3 cm).

              - Shall include Ø½ inch NPT female threaded bottom inlet.

              - Shall include a nozzle tree containing six nozzles with indicated flow rates from 0.54 to 4.6 gpm (2.0 to 17.4 l/m) and an exit trajectory of 10-25°.

              - All rotor body components shall be removable from the top without special tools in order to provide quick and easy flushing and maintenance of the sprinkler.

              - Shall include nozzle retention screw that may also be used to reduce spray radius by 25%.

              - Shall have a filter screen in the stem to protect the drive from clogging and to simplify removal for cleaning and flushing the system.

              - Shall utilize nozzles designed to deliver even distribution over the entire radius including large wind resistant droplets and gentle close-in-watering.

              - Shall include a three-year trade warranty.