      - SPRINKLERS

          - Description: Manufacture’s standard rotor designed to provide uniform coverage over entire area of spray shown on drawings at available water pressure or as follows:

              - Pop-up rotor for large turf areas (39-65 feet (11.9-19.8 m)) spacing: maximum 90 psi (6.2 bar).

          - Pop-Up rotor for large turf areas (39-65 feet (11.9-19.8 m)) spacing: maximum 90 psi (6.2 bar).  Irrigation rotor specifications include but are not limited to:

              - The rotor body, stem and nozzle, shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - Shall have a heavy-duty stainless steel retract spring for positive pop-down.

              - Shall have integrated seals and method for clearing debris.

                - Oversized pressure activated wiper seal prevents leaks and protects internals from debris.

                - Precision controlled flush at pop-down to clear debris from the unit, to assure positive stem retraction in all soil types.

              - Ratchet mechanism to allow easy arc alignment.

              - Full non-reversing 360° arc.

              - 8.5 inch (21.6 cm) body height; 4.0 inch (10.2 cm) pop-up height (measured to center of nozzle).

              - Operating range of 39 to 65 feet (11.9 to 19.8 m).

              - Operating pressure range of 30 to 90 psi (2.1 to 6.2 bar).

              - Shall have a precipitation rate of 0.37 to 1.31 in/hr (9 to 33 mm/hr).

              - Shall include check valve with up to 10 feet (3.0 m) of elevation hold up.

              - Shall include a stainless steel riser stem to deter vandalism on public turf areas and prevent scoring of stem from debris preventing leaks.

              - Exposed surface diameter shall measure 2.2 inch (5.5 cm).

              - Shall include Ø1 inch NPT female threaded bottom inlet.

              - Shall be available with eight nozzles with indicated flow rates from 2.9 to 21.7 gpm (10.8 to 82.2 l/m) and an exit trajectory of 25°.

              - All rotor body components shall be removable from the top without special tools in order to provide quick and easy flushing and maintenance of the sprinkler.

              - Shall include slot in cover for use of a pull up tool for ease of nozzle installation and replacement.

              - Shall include nozzle retention screw that may also be used to reduce spray radius by 25%.

              - Shall have a filter screen in the stem to protect the drive from clogging and to simplify removal for cleaning and flushing the system.

              - Shall utilize 3-port nozzles designed to deliver even distribution over the entire radius including large wind resistant droplets and gentle close-in-watering.

              - Shall include a five-year trade warranty.