      - IRRIGATION COMPONENTS AND/OR SYSTEMS

        - Rotor

          - General Information

            - Provide rotor as per the specification.

          - Rotor

            - Irrigation rotor specifications include but are not limited to:

              - The rotor swing arm and nozzle shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - The rotor body and barrel shall be constructed of aluminum.

              - Reversing 20-340° arc adjustment or full 360° rotation.

              - Operating range of 81 to 177 feet (25.6 to 57.3 m).

              - Operating pressure range of 30 to 120 psi (2.1 to 8.3 bar).

              - Shall include Ø2 inch NPT inlet.

              - Shall include a water deflector that uniformly distributes water for the entire throw distance.

              - Shall accept an optional jet breaker to improve distribution uniformity.

              - Shall include an automatic brake mechanism that allows the water jet to maintain a constant rotation speed in all operating conditions independent of the flow and pressure.

              - Shall include a rotation speed selector that has three rotation speeds to choose from.

              - Shall accept nozzles with indicated flow rates from 35 to 363 gpm (7.8 to 79.2 l/m) and an exit trajectory of 24°.

              - Shall include a one-year trade warranty.