      - IRRIGATION COMPONENTS AND/OR SYSTEMS

        - Rain Bird® 8000 Series Rotor

          - General Information

            - Provide Rain Bird® 8005NP as per the specification.

          - Rain Bird® 8005NP Rotor

            - Rain Bird® 8005NP rotor specifications include but are not limited to:

              - The rotor body, stem and nozzle, shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - Shall have a heavy-duty stainless steel retract spring for positive pop-down.

              - Shall have a brass reinforced turret for increased side impact durability.

              - Shall have a non-strippable drive mechanism to prevent damage from vandals.

              - Shall have the Rain Bird® Memory Arc™ feature to return the rotor to its original arc setting.

              - Shall have integrated seals and method for clearing debris.

                - Oversized pressure activated wiper seal prevents leaks and protects internals from debris.

                - Precision controlled flush at pop-down to clear debris from the unit, to assure positive stem retraction in all soil types.

              - Left and right side adjustable trips allow easy arc alignment.

              - Full and part circle in one unit. Reversing 50-330° arc adjustment through the top of the rotor requires only a flat blade screwdriver or non-reversing full 360° rotation.

              - 10.13 inch (25.7 cm) body height; 5.0 inch (12.7 cm) pop-up height (measured to center of nozzle).

              - Operating range of 39 to 81 feet (11.9 to 24.7 m).

              - Operating pressure range of 50 to 100 psi (3.5 to 6.9 bar).

              - Shall have a precipitation rate of 0.48 to 1.28 in/hr (12 to 31 mm/hr).

              - Shall include check valve (Rain Bird® Seal-A-Matic™ (SAM)) with up to 10 feet (3.0 m) of elevation hold up.

              - Shall include a stainless steel riser stem to deter vandalism on public turf areas and prevent scoring of stem from debris preventing leaks.

              - Exposed surface diameter shall measure 2.1 inch (5.4 cm).

              - Shall include a purple NP cover to indicate use for non-potable water.

              - Shall include Ø1 inch NPT female threaded bottom inlet.

              - Shall be available with twelve nozzles with indicated flow rates from 3.8 to 36.3 gpm (14.4 to 137.4 l/m) (See Figure) and an exit trajectory of 25°.

              - All rotor body components shall be removable from the top without special tools in order to provide quick and easy flushing and maintenance of the sprinkler.

              - Shall include slot in cover for use of a pull up tool for ease of nozzle installation and replacement.

              - Shall include nozzle retention screw that may also be used to reduce spray radius by 25%.

              - Shall have a filter screen in the stem to protect the drive from clogging and to simplify removal for cleaning and flushing the system.

              - Shall utilize 3-port Rain Bird® Rain Curtain™ nozzles with micro-ramp technology to deliver even distribution over the entire radius including large wind resistant droplets and gentle close-in-watering.

              - Shall include a five-year trade warranty.

              - Shall be manufactured by Rain Bird® Corporation, Azusa, California.

Nozzle Performance Chart