May 2017

Rain Bird® NimbusTM II and NimbusTM II LINK

Specifications for Bidding Purposes

Note to specifiers: These bidding specifications were current at the time of publication. They are subject to change without notice. Please confirm the accuracy of the specifications with Rain Bird or your local golf distributor.

The computerized central control system shall be the Rain Bird Nimbus™ II as hereinafter specified. It shall be capable of controlling three (3) independent, 18-hole golf courses, plus one “other” area, each consisting of greens, tees, fairways, approaches, perimeters, roughs and miscellaneous areas. The central shall include the Rain Bird Nimbus II Computer Assembly, a field hardware (satellite, decoder, or Integrated Control) interface, an uninterruptible power supply rated at 980 Watts or higher, a power circuit surge arrestor and a grounding network grid with surge arrestors, all as hereinafter specified.

All Nimbus II central control systems shall be “Hybrid” compatible. Nimbus II Hybrid computer systems shall have the flexibility to control up to three (3) field interfaces in any combination of interface type. Each of the interfaces will require a separate serial port for each device.  To enable the functionality of these items, a Nimbus II system Hybrid software module and additional interfaces (MIM, MIM-LINK or MIM-LINK900, ICI or LDI as needed) shall be supplied as specified on the drawings.

Nimbus II software shall include the following modules; Automatic Weather™, Smart Weather™, Rain Bird Messenger™, Hybrid Software, Smart Sensors™, The Freedom System™, Map Utilities, Map Layers and Multiple Weather Stations. An additional module purchase is required for Smart Pump™.

The Nimbus II system shall provide for the selection of three (3) different flow measurement units - U.S. gallons per minute (GPM), cubic meters per hour (M3/H) or liters per second (L/S). It shall also provide for the selection of any one of 22 different languages for display.

Nimbus II Software Capacity —The Nimbus II software shall operate in the Microsoft® Windows® 10 32 or 64-bit environment and shall be capable of controlling any one (1) or up to eight (8) of the four (4) types of Rain Bird field control systems: hard-wired satellite controllers; LINK or LINK900 radio-operated satellite controllers, decoder field devices, or IC System.

When used with field satellite controllers, each hard-wired or radio LINK type satellite interface shall be capable of controlling up to four (4) groups of satellites. Each wire group shall be capable of controlling up to 28 channels of 24 stations (max) providing a maximum of 672 stations per wire group. Using eight (8) MIM interface units (32 wire groups total) with the Hybrid software module, the Nimbus II system shall control a combined total of 21,504 stations with two-wire, LINK or LINK900 radio satellite controllers.

When used with a decoder system, Nimbus II software shall have the capacity to control a maximum of 4,000 single station decoders and activate up to 8,000 solenoids when using eight (8) LDI interfaces and a Hybrid software module.

The Nimbus II software, when used with an IC System, shall have the capacity to control a maximum of 24,000 single ICMs and activate up to 24,000 rotors or valves using eight (8) IC Interfaces and a Hybrid software module.

Continuous “real-time” communication - Between the central computer, interfaces and field satellite controllers, decoders or IC modules, the system shall provide continuous operation and response at all times. Continuous field “feedback” status information shall be registered in the computer software and also at the satellite interface when satellite systems are used. Nimbus II shall be a program/schedule-based system providing maximum flexibility of programming and giving the operator absolute and full control of the entire system. The Nimbus II system shall be capable of unlimited programs residing in the system at one time. Each program shall be further defined by a number of smaller schedules. A maximum of 50 programs and up to 50 schedules per program may be operated simultaneously. All programming shall be maintained in the computer memory and on the hard drive, from which they shall be executed. Programming shall NOT be “downloaded” to the field satellites. It shall NOT be possible to change or reprogram from the field thus assuring the operator full control at all times.

A “time window” may be defined for each individual program, confining its operation to this specific time period. Individual programs shall be capable of being designated for up to six (6) start times. Individual schedules shall be capable of being designated for up to 12 start times within the specified time window for their program. It shall be possible to designate the sequence of operation of areas and the sequence of operation of stations in these areas, within a given schedule.

Dynamic Flo-Manager® feature shall be included with the software and automatically distribute and limit flow within the system, to eliminate hydraulic “overload” while maintaining maximum system operating efficiency, without the requirement of entering flow zone or branch piping data. The system shall also be capable of entering complete flow management database information for up to six (6) independent pump stations; up to 250 piping network branches and up to 999 flow zones for each pump station. This shall result in the highest efficiency of pump station operation, shortest watering cycle time and conservation of energy. During operation, individual flow graphs shall be automatically generated for each of the three (3) courses, with individual station activity information being presented in colorful charts. Flow graphs shall be automatically stored in the software for future access and reference.

The Nimbus II Smart Weather™ optional software module, when specified, shall monitor and respond to climatic conditions as they occur by tracking evapotranspiration (ET) rates and other sensory inputs from up to five (5) on-site weather stations. Smart Weather shall also log weather conditions for future reference. Smart Weather shall provide automatic response from user- defined thresholds on up to five (5) WS-PRO2 Weather Stations. The Smart Weather responses shall be provided to the computer for programmed response and shall be capable of sending an email or text message to the user for alarm conditions with the optional Rain Bird MessengerTM module.

Pump Profiling™ - Nimbus II shall provide user-definable limits for irrigation system capacity to manage system flow and decrease power consumption during peak electrical periods. If so desired, the user shall define irrigation system capacity for each hour of the day to optimize system efficiency according to electrical demand. The Nimbus II software shall automatically increase or decrease system capacity according to these user-defined limits.

Programming Flexibility - The Nimbus II system shall also provide for programs to be set to adhere to manual water budgeting; at the system level, at the individual program level and/or at the individual schedule level. A watersaver feature shall provide water budgeting capabilities from zero (0) to 300% in one (1) % increments. Automatic rain shutdown shall be possible with the integration of a rain sensor.

An innovative, guided initialization and “start-up” programming method in Nimbus II shall result in a customized QuickStart™ program that gets the system “up” and “operating” in the shortest possible time. Built-in rotor database tables shall provide for easy specification of station sprinklers for custom irrigation scheduling. Precipitation rates for each station shall be automatically calculated with the selection of sprinkler model, pattern and spacing.

A unique QuickIRR™ method of programming shall provide for a quick and easy method to automatically build programs to meet all irrigation challenges and allow programming by specific areas and designating sequence of operation of these areas. This feature is enhanced in Nimbus II by providing the ability to program multiple courses. Select the course, hole, area, sequence and run time and Nimbus II will calculate the most efficient irrigation sequencing.

A “Dry Run” feature shall provide for simulation of a program prior to operation, enabling the user to make the necessary adjustments before actually operating it in the field. A printout of the “dry run” results shall be possible, as well as being displayed on the monitor.

The Nimbus II Cycle + Soak™ feature works with Flo-Manager to achieve maximum efficiency and conservation. It helps control water application on slopes and in areas with poor drainage. Cycle + Soak shall maintain pump station demand while preventing over application in challenging areas of the course.  The Cycle + Soak feature will ensure the maximum cycle time defined by the user is never exceeded and will not change with changes in station runtime.

Graphical display of the golf course can be achieved using GPS, CAD, aerial photograph or the Scorecard function.  Each hole can be defined to indicate the areas to be irrigated such as greens, tees, fairways, approaches, perimeters, roughs and miscellaneous areas. The system shall provide for multi-station programming and operation of individual stations. A station data table shall give complete database information for each individual station.

The FREEDOM SystemTM - The Nimbus II system shall be capable of direct manual access of any station at any time. Full system remote control via handheld radio or remote telephone commands shall be possible with the integration of The FREEDOM System™ and handheld software module.

Logs and Troubleshooting - The Nimbus II system shall provide for an individual course, daily and seasonal logs for record keeping and easy compliance with regulatory requirements regarding water usage. A unique “Cost Estimator” feature shall provide projections of water and power costs for specific irrigation cycles, which can be used in establishing budget requirements.

The Nimbus II decoder-based system shall provide an automatic decoder and line condition testing program, for easy check-out and troubleshooting of the system.

When used with an Integrated Control System, the Nimbus II software shall contain detailed diagnostics software that measures a series of parameters related to system operation. These parameters shall include ICI and ICM operation. The software diagnostics shall be capable of polling individual station ICM’s on each wire path and display the number of passed and failed ICMs on each wire path. The diagnostics shall also be capable of measuring the total mA draw on each wire path. Individual ICM’s can be interrogated to confirm communication with the central control software as well as measure the voltage at each ICM for troubleshooting purposes. The diagnostics shall be capable of communicating with each ICM by “fast connect” or through the “long address” setting based on the diagnostic function.

Hardware—Computer - Furnish and install at the central location a Rain Bird Nimbus II computer assembly consisting of the following minimum specifications:

DELLTM OptiplexTM 7040 Mini Tower

Intel® Core™ i5-6500 Processor

8GB, DDR4 RAM

1TB  Solid State Drive

US English (QWERY) Wireless Keyboard and Mouse8X DVD+/-RW Optical Disk Drive

Intel® Integrated Graphics

24" Widescreen HD LED-Lit Flat-Panel Monitor

16 GB USB Flash Drive

6 Serial (RS232/COM) Ports

1 Parallel Port

10 External USB 2.0 Ports (6 USB 3.0, 4 USB 2.0)

Preinstalled software shall consist of:

Rain Bird Nimbus II software program

Microsoft Windows® 10 Professional 64-bit

Optional software modules:

SmartPump™ Software Module

Voltage Stabilizer (120VAC)—At the central location, furnish and install a combination voltage stabilizer and uninterruptible power source. Unit shall have a minimum rated output of 1440VA and 980 Watts. It shall be suitable for 50/60 Hz operation with input power of 120VAC. The unit shall operate in the AC mode from 82VAC input up to 144VAC input, regulating the output voltage within proper limits. Transfer to battery mode shall occur at any input voltage less than 75VAC or greater than 154VAC. In battery mode, output shall be a pure Sine wave form. Stepped or approximated Sine wave forms shall not be acceptable. Output voltage regulation shall be less than 5% at full load. Frequency regulation shall be +/- 3 Hz on battery. Surge energy rating shall be a minimum of 459 Joules. Battery back-up shall have a minimum time of approximately 14 minutes minimum at half-load capacity. The unit shall have Quick Status Indicators and an LCD menu-driven display screen showing power status, control settings, configuration, test and diagnostics, and logs. USB and Smart-Slot computer interface ports shall enable communication with the central control computer. The tower housing shall have a minimum of eight (8) “battery and surge” NEMA 5-15R electrical outlets. The voltage stabilizer shall be the APC Smart-UPS 1500 or higher.

Voltage Stabilizer (220-240VAC)— At the central location, furnish and install a combination voltage stabilizer and uninterruptible power source unit. Unit shall have a rated output of 1500VA and 980 Watts. It shall be suitable for 50/60 Hz operation with input power of 230VAC. The unit shall have configurable nominal output voltage of 220VAC, 230VAC, or 240VAC. At 230VAC, the unit shall operate in the AC mode from 151VAC input up to 302VAC input, regulating the output voltage within proper limits. Transfer to battery mode shall occur at any input voltage less than 160VAC or greater than 286VAC. In battery mode, output shall be a pure Sine wave form. Stepped or approximated Sine wave forms shall not be acceptable. Output voltage regulation shall be less than 5% at full load. Frequency regulation shall be +/- 3 Hz on battery. Surge energy rating shall be a minimum of 480 Joules. Battery back-up shall have a minimum time of approximately 14 minutes minimum at half-load capacity. The unit shall have Quick Status Indicators and an LCD menu-driven display screen showing power status, control settings, configuration, test and diagnostics, and logs. USB and Smart-Slot computer interface ports shall enable communication with the central control computer. The tower housing shall have a minimum of eight (8) “battery and surge” NEMA 5-15R electrical outlets. The voltage stabilizer shall be the APC Smart-UPS 1500VA-230V or higher. The manufacturer shall verify that the voltage stabilizer is approved for use in the local country where the unit is to be installed.

Power Surge Arrestor—At the main electrical panel and on the circuit supplying the central equipment. Furnish and install a Model “Z1” Zap Trap surge arrestor. Unit shall be for 120/240 VAC, single-phase power rated for 100 Amps. It shall have a discharge capacity of 15,000 Amps at an 8 x 20 second pulse. It shall have a clamping voltage of 130/250 VAC and a response time of 1.5 N/sec. Surge arrestor shall be as manufactured by Tytewadd Power Filters, Springfield, Missouri 65807, phone: 417-887-3770.

System Grounding Grid—At the central control location, as close to the Interface as possible, install a 3-rod grounding grid. This shall consist of three (3) - 5/8" (1.6 cm) diameter x 8' (2.4 m) long copper or copper clad ground rods arranged in a linear pattern with each rod at least 16' (4.8 m) away from any other rod. The rods shall be tied together below ground using #6 gauge or larger bare copper wire and brass clamps. A separate clamp shall be used for each attachment. Rods shall be driven into the ground with top 6" (15.2 cm) below the finish grade. Install a standard 12" x 18" x 12" (30.5 cm x 45.7 cm x 30.5 cm) rectangular valve box over the top of any rods connected by a grounding wire to a surge arrestor, the grounding lug of a piece of equipment or an MGP-1 grounding plate assembly. This shall provide future access to inspect and/or maintain it properly. Other rods shall have a standard 6" (15.2 cm) diameter round valve box and cover installed over the top of the rod for future access.

A #6 gauge or larger bare copper ground wire shall be run from the grounding lug of the interface and attached to one rod of this 3-rod ground grid. On each two-wire path, coming from the interface and going out to the field, furnish and install an MSP-1 surge arrestor, which is to be securely mounted on an MGP-1 grounding plate assembly. The MGP-1 shall be securely attached to one ground rod by means of the “U” clamps provided. Connect the MSP-1 arrestor to the two-wire path according to the manufacturer’s specifications. An earth ground resistance of 10 OHMS or less shall be maintained at the grounding grid. Recommended resistance of 5 OHMS or less is preferred for optimum ground resistance.

Communication Wire—(hard-wired satellite system, decoder based system and Integrated Control System) — Furnish and install, for the two-wire communication paths, double jacketed type wire, consisting of two (2) tin-coated type UF insulated (4/64" PVC), soft drawn, annealed solid copper conductors. The two (2) conductors shall be color-coded (one RED the other BLACK).

The second outer jacket shall be a solid color, high density, polyethylene insulation. Jacket colors shall be different for each wire path on the interface. Jacket colors and conductor sizes shall be as shown on the drawings and/or as directed.

Note to specifiers:

Additional specifications to complete your documentation can be found on the Rain Bird Golf Specifier web site. Additional specifications include:

- Rain Bird Sprinklers

- Rain Bird Field Hardware (interfaces and field hardware)

- Rain Bird Weather Stations

- The FREEDOM System

- Rain Bird Valve Boxes

- Rain Bird Pump Stations

- Rain Bird Swing Joints

- Rain Bird Integrated Sensor System (ISS)