      - SPRINKLERS

          - Description: Rain Bird® 5000 Series Rotor designed to provide uniform coverage over entire area of spray shown on drawings at available water pressure or as follows:

              - Rain Bird® 5006+PCSAMRSS Rotor for shrub or small turf areas (25-50 feet (7.6-15.2 m)) spacing: maximum 65 psi (4.5 bar).

          - Pop-Up rotor for shrub or small turf areas (25-50 feet (7.6-15.2 m)) spacing: maximum 65 psi (4.5 bar).  Irrigation rotor specifications include but are not limited to:

              - The rotor body, stem and nozzle, shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - Shall have a heavy-duty stainless steel retract spring for positive pop-down.

              - Shall have integrated seals and method for clearing debris.

                - Oversized pressure activated wiper seal prevents leaks and protects internals from debris.

                - Precision controlled flush at pop-down to clear debris from the unit, to assure positive stem retraction in all soil types.

                - Ported arc adjustment screw to clear debris from unit at pop-down

              - Shall have a Rain Bird® slip clutch installation feature to allow easy arc alignment.

              - Reversing 40-360° arc adjustment through the top of the rotor requires only a flat blade screwdriver.

              - 9.63 inch (24.5 cm) body height; 4.0 inch (10.2 cm) pop-up height (measured to center of nozzle).

              - Operating range of 25 to 50 feet (7.6 to 15.2 m).

              - Operating pressure range of 15 to 55 psi (1.0 to 3.8 bar).

              - Shall have a precipitation rate of 0.20 to 1.50 in/hr (5 to 37 mm/hr).

              - Shall include a green cover and a flow shut-off valve in the rotor head.

              - Shall include check valve (Rain Bird® Seal-A-Matic™ (SAM)) with up to 7 feet (2.1 m) of elevation hold up.

              - Shall include a pressure regulating stem (PRS) that regulates pressure to 45 psi (3.1 bar).

              - Shall include a stainless steel riser stem to deter vandalism on public turf areas and prevent scoring of stem from debris preventing leaks.

              - Exposed surface diameter shall measure 1.5 inch (3.8 cm).

              - Shall include Ø3/4 inch NPT female threaded bottom inlet.

              - Shall include a nozzle tree containing twelve nozzles with indicated flow rates from 0.76 to 9.63 gpm (3.0 to 36.6 l/m) (See Figure) and an exit trajectory of 10-25°.

              - All rotor body components shall be removable from the top without special tools in order to provide quick and easy flushing and maintenance of the sprinkler.

              - Shall include slot in cover for use of a pull up tool for ease of nozzle installation and replacement.

              - Shall include nozzle retention screw that may also be used to reduce spray radius by 25%.

              - Shall have a filter screen in the stem to protect the drive from clogging and to simplify removal for cleaning and flushing the system.

              - Shall utilize Rain Bird® Rain Curtain™ nozzles with micro-ramp technology to deliver even distribution over the entire radius including large wind resistant droplets and gentle close-in-watering.

              - Shall include a five-year trade warranty.

              - Shall be manufactured by Rain Bird® Corporation, Azusa, California.

Nozzle Performance Chart