      - IRRIGATION COMPONENTS AND/OR SYSTEMS

        - Rain Bird® Maxi-Paw Series Rotor

          - General Information

            - Provide Rain Bird® 2045ASAM Maxi-Paw as per the specification.

          - Rain Bird® 2045ASAM Maxi-Paw Rotor

            - Rain Bird® 2045ASAM Maxi-Paw rotor specifications include but are not limited to:

              - The rotor case, cover and nozzle, shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - Shall have a heavy-duty stainless steel retract spring for positive pop-down.

              - Shall have method for clearing debris.

                - Multi-function pressure activated wiper seal prevents leaks and protects internals from debris.

                - Self-flushing inner trip.

              - Clamp mechanism allows easy arc alignment.

              - Reversing 20-340° arc adjustment or full 360° rotation.

              - 9.3 inch (23.6 cm) body height; 3.0 inch (7.6 cm) pop-up height (measured to center of nozzle).

              - Operating range of 22 to 45 feet (6.7 to 13.7 m).

              - Operating pressure range of 25 to 60 psi (1.7 to 4.1 bar).

              - Shall have a precipitation rate of 0.28 to 1.21 in/hr (7 to 31 mm/hr).

              - Exposed surface diameter shall measure 5.0 inch (12.7 cm).

              - Shall include check valve (Rain Bird® Seal-A-Matic™ (SAM)) with up to 10 feet (3.1 m) of elevation hold up.

              - Shall include Ø½ inch NPT female threaded side inlet and a combination Ø½ inch and Ø3/4 inch bottom inlet.

              - Shall include seven nozzles with indicated flow rates from 1.5 to 8.4 gpm (5.7 to 31.8 l/m) (See Figure) and an exit trajectory of 11-23°.

              - All rotor body components shall be removable from the top without special tools in order to provide quick and easy flushing and maintenance of the sprinkler.

              - Shall include a double weighted horizontal oscillating arm to provide smooth, slow rotation and increase distance of the throw.

              - Shall include distance diffuser screw that may be used to reduce spray radius by 25%.

              - Shall have a filter screen in the bottom to protect the nozzle from clogging and to simplify removal for cleaning and flushing the system.

              - Shall utilize nozzles designed to deliver even distribution over the entire radius.

              - Shall include a three-year trade warranty.

              - Shall be manufactured by Rain Bird® Corporation, Azusa, California.

Nozzle Performance Chart