      - IRRIGATION COMPONENTS AND/OR SYSTEMS

          - Description: Valves are designed to provide reliable and convenient manual water access or as follows:

              - Valve for small turf areas (30 gpm (113.6 l/min)): maximum 125 psi (8.6 bar).

          - Manual valve for small turf areas (30 gpm (113.6 l/min)): maximum 125 psi (8.6 bar).  Irrigation valve specifications include but are not limited to:

              - The valve body shall be constructed of brass.

              - The valve cover shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - The body shall be constructed as a two-piece design for easy servicing.

              - Shall contain a strong corrosion resistant stainless steel spring to prevent leakage.

              - The valve shall be activated using a separate QCV key that is inserted into the valve body to provide water access.

              - The valve shall be connected to the irrigation system using Ø1” female NPT pipe thread.

              - The valve key shall have Ø ¾” female ACME connection to mate to the valve body and a Ø1” male NPT pipe thread to connect to an external hose or equivalent.

              - The valve cover shall include a locking mechanism that engages the track in the body to prevent cover from being opened.

              - The valve cover shall be purple and marked with “Do Not Drink” warnings in English and Spanish for use with non-potable water source.

              - Operating flow rate up to 30 gpm (113.6 l/min).

              - Operating pressure range of 5-125 psi (0.3-8.6 bar).

              - Water temperature: Up to 110°F (43°C).

              - Ambient temperature: Up to 125°F (52°C).

              - Shall include a three-year trade warranty.