      - IRRIGATION COMPONENTS AND/OR SYSTEMS

        - Rain Bird® QCV Series Valve

          - General Information

            - Provide Rain Bird® 33DLRC as per the specification.

          - Rain Bird® 33DLRC Valve

            - Rain Bird® 33DLRC valve specifications include but are not limited to:

              - The valve body shall be constructed of brass.

              - The valve cover shall be constructed of heavy-duty and ultra-violet resistant plastic.

              - The body shall be constructed as a two-piece design for easy servicing.

              - Shall contain a strong corrosion resistant stainless steel spring to prevent leakage.

              - The valve shall be activated using a separate QCV key that is inserted into the valve body to provide water access.

              - The valve shall be connected to the irrigation system using Ø¾” female NPT pipe thread.

              - The valve key shall have Ø½” female connection to mate to the valve body and a Ø¾” male NPT pipe thread to connect to an external hose or equivalent.

              - The valve cover shall include a locking mechanism that engages the track in the body to prevent cover from being opened.

              - Operating flow rate up to 20 gpm (75.7 l/min).

              - Operating pressure range of 5-125 psi (0.3-8.6 bar).

              - Water temperature: Up to 110°F (43°C).

              - Ambient temperature: Up to 125°F (52°C).

              - Shall include a three-year trade warranty.

              - Shall be manufactured by Rain Bird® Corporation, Azusa, California.

Valve Pressure Loss Chart